# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Supply of ambulance vehicle for the project Be-Healthy

Ref. no. HR-RS00250-4/ambulance vehicle

Columns 1-2 should be completed by the Project partner

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Project partner shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The requirements set out in the technical specifications represent the minimum technical characteristics which offered goods must satisfy, unless stated otherwise, and tenderers are not allowed to modify technical specification in any way.

For each item for which it is not explicitly stated that it is allowed to offer goods of the equal characteristics, i.e. for each item where it is not stated “or equivalent”, for the purposes of this tender documentation it is assumed that words “or equivalent” are stated, and tenderer is allowed to offer equivalent goods / goods of equivalent characteristics.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s decision (Y/N) |
| --- | --- | --- | --- | --- |
| 1 | Ambulance vehicle with next characteristics – 1 pc: - Motor: diesel, EURO 6 - Motor volume: min. 2100 cm3 - Transmission: manual 6 gears - Motor power: min. 100 kw - Useful load capacity: min. 1000 kg - Gross carrying capacity: 3500 kg - Vehicle length: min 5000 mm - Wheelbase: min. 3000 mm - Number of doors: 4 - Number of seats: 3 - Vehicle must be equipped with standard equipment: ABS, ESP, BA, EBV, Cruise control, TPMS, Hill Start Assist, Start/Stop system, Rear parking sensors, Audible signal to warn pedestrians when reversing, Active lane keeping assist system, - Recognition of speed limit traffic signs, min. 3" trip computer display, Cup holder on the central console - Halogen headlights, Daytime running lights, - Right side sliding door without glass - Double rear doors without glass with opening up to 180°, - Rear bumper step with anti-slip coating - Wheels with steel rims 15", tire 215/70 R 15 109S, - Full size steel spare wheel |  |  |  |
|  | Interior - The interior of the patient area is completely lined with polyester panels with a combination of ABS plastic with blue decorative moldings, the panels are adapted to the radii of the vehicle. - polyester castings with polyurethane filling are resistant to all detergents and disinfectants on a chemical or biological basis - acid-resistant. - formwork with thermo-sound insulation ensures the K factor of heat transfer K < 70 W /m2 C. The level of heat-sound insulation ensures a pleasant stay and significantly reduces the loss of energy for heating or cooling the space. - the formwork is adjusted according to the additional equipment. - polyester castings are made with self-extinguishing material and have a burning speed of 10mm/min. |  |  |  |
|  | Electrical installation The installation of the 12V lines was carried out separately from the battery independent of the standard installation of the vehicle, all installation was carried out through fixed bulletproof hoses. Each connection and consumer in addition to the main fuse is secured by a separate fuse easily accessible on the control panel. 12V power sockets are installed External connector for 220V on the front on the driver's side and 2 sockets 220V inside |  |  |  |
|  | Independent heating of the examination area An independent diesel heater that works independently of the engine and uses fuel from the battery. Electric heater 2.2 KW, 220V that works when the vehicle is connected to the external network Ventilation A two-way fan installed in the ceiling light on the roof, covered with a polyester casing, enables air to be blown in and out through a micro filter into the patient area with 30 air changes per hour. Ventilation is switched on from the control panel in the patient area. |  |  |  |
|  | Lighting of the patient area The vehicle is lit by LED lighting on the ceiling.. Above the front door are built-in lights with a timer that are activated when the door is opened. All lights are switched on independently from the control panel in the patient area. Vehicle marking Marking of the vehicle is carried out by complete branding according to the tender request with conceptual solutions of the client. The windows in the patient area are tinted with foil with one-sided visibility. Body Conversion and Interior Equipment Requirements -Thermal insulation shall be installed beneath the polyester wall and ceiling panels. -The interior shall be lined with washable polyester panels to create a seamless, easy-to-clean surface throughout the compartment. - A non-slip floor covering shall be installed over a waterproof plywood subfloor. - An air-conditioning evaporator 6KW unit connected to the vehicle’s original air-conditioning system shall be installed in the rear compartment. - An independent diesel-powered heater with a minimum output of 5 kW shall be installed in the rear compartment. - An electric heater rated at 2.2 kW, 220V AC, shall be installed and operational when the vehicle is connected to an external power source. - One external 220V AC power inlet and a minimum of two internal 220V AC power outlets shall be provided. - Complete electrical distribution system including 12V DC and 220V AC circuits, USB charging outlets, thermometer, and voltmeter shall be installed. - A roof-mounted 2-way ventilation fan with a minimum power consumption of 40W and a minimum airflow capacity of 450 m³/h shall be provided. - Vehicle markings and graphics shall be applied according to the Purchaser’s requirements. - All windows in the patient compartment shall be fitted with one-way privacy tinted film. - The interior shall be illuminated by ceiling-mounted LED lighting. - All interior lights shall be independently controlled from a control panel located in the patient compartment. - Waste bin. - Grab handles at the entrance doors. - Fire extinguisher. - One patient transport chair constructed from lightweight aluminum. - The chair shall be equipped with a minimum of two (2) wheels and four (4) foldable carrying handles. Dimensions: Depth: min. 480 mm Width: min. 480 mm Height: min. 900 mm Minimum load capacity: 150 kg Maximum chair weight: 8 kg Set of winter tires with steel rims |  |  |  |
|  | Conversion Layout Description Section 1 - Two (2) anatomically shaped seats with high backrests, upholstered in blue artificial leather, equipped with armrests and three-point seat belts, mounted against the front left side wall. - One work desk manufactured from materials resistant to all standard cleaning and disinfecting agents, equipped with three drawers. Approximate dimensions: min. 350 mm × 700 mm × 400 mm. Section 2 Patient Seating and Examination Area - One patient examination couch mounted along the left side of the vehicle. - Two side-facing seats mounted against the right-side wall of the vehicle, without seat belts and not considered passenger seating positions. - A table with one drawer shall be installed between the side seats on both sides. Medical Storage Cabinets - Storage compartments for medical supplies shall be installed on both the left and right sides above the seats. - The compartments shall be integrated into the vehicle structure and fitted with sliding shutter doors for secure storage of medical equipment and consumables. |  |  |  |
|  | A valid decision of the Agency for Medicines and Medical Devices of Serbia (ALIMS) by which the offered medical device is registered or approved for placing on the market of the Republic of Serbia. If the tenderer is not the manufacturer of the medical device, tenderer is obliged to submit a valid authorization from the manufacturer or its authorized representative confirming that tenderer is authorized to sale and deliver the offered medical device on the territory of the Republic of Serbia. |  |  |  |
|  | Delivery and installation Equipment must be delivered to Public Health Institute Sombor, Vojvođanska 47, 25000 Sombor |  |  |  |
|  | Vehicle upgrade certificate Ambulance vehicle must be certificated in line with traffic regulations and laws in Republic of Serbia |  |  |  |
|  | Warranty period Min. warranty period is 24 months from the date of delivery |  |  |  |